INDIAN INSTITUTE OF TECHNOLOGY, BOMBAY

Department of Metallurgical Engineering and Materials Science

MM 202: THERMODYNAMICS : 2018-19: Fall

Tutorial No. 7:          				                                                                         Date: Sept 18, 2018

- (a) 100 g of liquid iron-manganese solution, at 1873K, is in equilibrium with 20 g of a FeO-MnO liquid solution with XMnO = 0.1.  The total pressure is 1 atm. MnO and FeO liquid solution is ideal, when the respective standard states are pure liquids (even if metastable).

- Calculate percentage Mn dissolved in iron and

- pO2 in the gas phase

Mn may be assumed to follow Henry’s law in iron, with oMn = 1.30 at 1873K.

FeO liquid is stoichiometric.

[HINT: O2 should be in equilibrium simultaneously with Fe-FeO and Mn-MnO systems].

- If the gas phase also contained H2 and H2O, what is the pH2/pH2O ratio